Executive Summary

Development of Bidirectional Supercapacitor Power Management Board for

RMUC Sentry robot

Project Summary

The Robomaster University Championship (RMUC) challenges university teams to design advanced robots with enhanced power management capabilities to work with the limited power available. This project focuses on developing a supercapacitor board with bidirectional charging and discharging capabilities to support peak power demands of the sentry robot during intense competition. Key features to maximize the power source available include programmable bidirectional current limiting for effective load sharing between supercapacitors and batteries, as well as simultaneous charging of supercapacitors and powering of the robot under constant power constraints. An efficient power booster pack Switch Mode Power Supply (SMPS) utilizing a four-switch bidirectional buck-boost topology was developed. The design process encompassed circuit simulation, prototyping, and load testing, ensuring a robust solution tailored to RMUC's unique demands.

Key Developments

- 4-Switch Bidirectional Buck-Boost DC-DC SMPS:

Sentry robot requires a SMPS capable of handling wide input and output voltage variation, including scenarios where Vin > Vout, Vin < Vout and Vin = Vout, as well as supporting bidirectional current flow, a 4-Switch Buck-Boost topology is chosen to allow buck, boost and buck-boost modes in either direction to charge the supercapacitor bank or provide power to the robot.

- Optimized Current Sharing:

During operation, power draw of sentry robot is dynamic, drawing little current while idling but peaking to 400W during fast movements. To keep within a static power limit imposed by competition rules, the SMPS must charge the supercapacitor while sentry has excess current capacity to spare when idling, or supply enough current to satisfy peak loads during fast reactions. Responsiveness of the control loop is critical to keep up with dynamic load variations and prevent competition power limit overshoot.

- Fabrication and Results Verification:

A functional prototype is fabricated and tested for various parameters including load step response, total efficiency, effective capacity and stress testing to simulate harsh conditions during competition. Characteristics of the SMPS is verified to meet competition requirements and durability.

Challenges and Limitations

Although load regulation is measured to meet specifications with load step tests, some challenges remain in the form of unstable inner current mode control loop, likely due to noise from noisy power traces like MOSFET gate drive traces coupling into small signal sense traces running parallel to the gates without adequate shielding. These inner loop instabilities result in slightly lower efficiency (measured 90% instead of >95% nominal) which will slightly impact performance.

Results and Contributions

This project successfully enhanced the Sentry robot's maximum performance and responsiveness to events with a power booster pack to increase peak current capability enhancing the movement speed of the sentry. These improvements position the robot to better meet the demands of RMUC, setting a foundation for continued optimization and competitive performance.

Future Directions

Future improvements to the power buffer focus on enhancing loop stability and enabling digital control integration. Stability issues observed during transients highlight the need for refined compensation and improved PCB design to ensure reliable operation. Additionally, developing a digital interface board will allow real-time configurability and smarter power management through integration with the robot’s main controller.